LV618 IPSL Handover Document

5GC KDDI Kyanos Project – AUSF_UDM | CSD | NPC

| Author | Rohit Rathod |
| --- | --- |
| Organization | Nokia |
| Document Type | Initial Handover Documentation |
| Document Version | 1.0 |
| Date | 2026-02-18 |
| Classification | Internal / Customer Sharing |

# Version History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | 2026-02-18 | Rohit Rathod | Initial handover document creation |

Contents

1.Version History	2

2. Introduction	4

3. Objective	4

4. Scope of Work	4

4.1 IPSL Server Setup	4

4.2 Script Migration	5

4.3 Test Case Documentation and Execution	5

5.Environment Details	5

6.Test Case Details	5

6.1 CSD Functional Test Case Details	5

6.2 DRA Functional Test Case Details	6

6.3 BSF Functional Test Case Details	6

6.4 Integration Test Case Details	6

6.5 Overload Handling Test Case Details	6

7.Test Case List	7

7.1 CSD Functional Test Cases	7

7.2 DRA Functional Test Cases	9

7.3 BSF Functional Test Cases	11

7.4 Integration Test Cases	12

7.5 Overload Handling Test Cases	13

8.Conclusion	14

# 2. Introduction

This document constitutes the formal technical handover record for the IPSL server setup, script migration, and functional/integration test case execution activities conducted within the Nokia KDDI 5G Core (5GC) project environment.

The activities covered herein span server provisioning and assignment, IPSL software installation, static route configuration, legacy script migration from the SCP23.8 project environment, and the preparation and documentation of test cases for the CSD, DRA, BSF, Integration, and Overload Handling domains.

# 3. Objective

The objectives of this engagement are as follows:

- Provision and configure Windows servers for IPSL within the AWS lab environment.

- Install IPSL tool on designated Windows servers and apply required static route configurations per Vendor Lab LLD.

- Migrate existing IPSL scripts from the SCP23.8 project servers (10.1.52.7 and 10.1.52.8) to the newly provisioned IPSL servers.

- Establish a prioritized migration plan: Priority 1 – Windows 01, 03, 05; Priority 2 – Windows 02, 04, 06.

- Document, structure, and maintain the test case repository for CSD Functional, DRA Functional, BSF Functional, Integration, and Overload Handling test areas.

# 4. Scope of Work

The following activities are within scope of this handover:

## 4.1 IPSL Server Setup

- AWS lab account creation and PC setup.

- IPSL software download and installation on all six Windows servers.

- Static route configuration per Vendor Lab LLD for each server.

## 4.2 Script Migration

- Identify source script locations on 10.1.52.7 and 10.1.52.8.

- Copy all files from IPSL (10.1.52.7) and (10.1.52.8) to target servers.

- Validate migrated scripts on each target server.

## 4.3 Test Case Documentation and Execution

- CSD Functional test cases

- DRA Functional test cases

- BSF Functional test cases

- Integration test cases

- Overload Handling test cases

- AUSF_UDM test cases

- NPC test cases

# Environment Details

The following table describes all servers and systems utilized during this engagement:

| Component | Purpose | IP / Identifier |
| --- | --- | --- |
| Windows Server 01 | IPSL – Priority 1 | AUSF_UDM |
| Windows Server 02 | IPSL – Priority 2 | AUSF_UDM |
| Windows Server 03 | IPSL – Priority 1 | CSD |
| Windows Server 04 | IPSL – Priority 2 | CSD |
| Windows Server 05 | IPSL – Priority 1 | NPC |
| Windows Server 06 | IPSL – Priority 2 | NPC |

# Test Case Details

## 6.1 CSD Functional Test Case Details

- The CSD functional test cases cover SBI traffic handling for N7, N10, and related interfaces.

- Header-based routing using 3gpp-Sbi-Target-apiRoot.

- N7 create, update, and delete operations for PCF and s-PCF subscribers.

- Mated ME Pair failover scenarios (ME1 down, both MEs down, CSD instance failover).

- Failure scenarios returning 504 Gateway Timeout or 503 Service Unavailable.

## 6.2 DRA Functional Test Case Details

- DRA test cases cover Gx and Rx Diameter interface routing through the n-DRA and DRA layers to k-NPC Mated ME Pairs. Key scenarios include:

- Gx CCR-I/U/T routing with SLF/RSV lookup and DH AVP-based routing.

- Failover: single k-NPC down, all k-NPCs down, all DRA down, all n-DRA down.

## 6.3 BSF Functional Test Case Details

- BSF test cases validate the binding session lifecycle and NRF registration/deregistration behaviour:

- BSF Create: NPCv1 sends Nbsf create via SCP

- BSF Delete: session removed upon delete request.

- NRF Registration, De-Registration, and Heartbeat (PATCH) verified via Grafana dashboard.

- Session expiry verified via IntitalContextExpiry timer.

## 6.4 Integration Test Case Details

Integration test cases verify end-to-end connectivity and data flows across all integrated components:

## 6.5 Overload Handling Test Case Details

Overload handling test cases verify alarm escalation, traffic rejection, and system recovery under high-load conditions. All eight test cases are Priority 2 and require KDDI support for traffic load generation. Alarm severity progression: Minor -> Major -> Critical, with corresponding clearing behavior.

# Test Case List

## 7.1 CSD Functional Test Cases

| Test Case ID | Test Topic | Description |
| --- | --- | --- |
| E00001 | N10 Header base routing | Verify message routing via 3gpp-Sbi-Target-apiRoot header |
| E00002 | N7 create – PCF data | SM Policy Association Establishment for data service (PCF subscriber) |
| E00003 | N7 update – PCF data | SM Policy Association Update for data service (PCF subscriber) |
| E00004 | N7 delete – PCF data | SM Policy Association Termination for data service (PCF subscriber) |
| E00005 | N7 create – dnn=ims (Negative) | SM Policy Association Establishment for voice service – returns 503 |
| E00006 | N7 create – s-PCF data | SM Policy Association Establishment for data service (s-PCF subscriber) |
| E00007 | N7 update – s-PCF data | SM Policy Association Update for data service (s-PCF subscriber) |
| E00008 | N7 delete – s-PCF data | SM Policy Association Termination for data service (s-PCF subscriber) |
| E00009 | N7 create – unregistered SLF | SM Policy Association Establishment – unregistered SLF subscriber |
| E00010 | N7 create – PCF (ME Pool) | SLF Destination ID belongs to k-NPC Destinations ME Pool (Ticket 30391) |
| E00011 | N7 create – PCF / ME1 down | Mated Pair1 ME1 down; route to ME2 (Ticket 30527) |
| E00012 | N7 create – PCF / both MEs down | Both MEs of Mated Pair1 down; returns 504 Gateway Timeout |
| E00013 | N7 update/delete – PCF / ME1 down | Update/Termination; ME1 down, route to ME2 |
| E00014 | N7 update/delete – PCF / both MEs down | Update/Termination; both MEs down; returns 504 |
| E00015 | N7 create – s-PCF / ME1 down | s-PCF subscriber; ME1 down, route to ME2 |
| E00016 | N7 create – s-PCF / both MEs down | s-PCF subscriber; both MEs down; returns 504 |
| E00017 | N7 update/delete – s-PCF / ME1 down | s-PCF update/termination; ME1 down, route to ME2 |
| E00018 | N7 update/delete – s-PCF / both MEs down | s-PCF update/termination; both MEs down; 504 timeout |
| E00024 | N10 notify – UDM1 down | UDM1 down; returns 504 Gateway Timeout (Failure Scenario) |
| E00025 | N10 notify – SMF1 down | SMF1 down; returns 504 Gateway Timeout (Failure Scenario) |
| E00026 | N7 create – CSD-1 down | CSD-2 handles request; PCF subscriber; CSD-1 down |
| E00027 | N7 create – CSD-1 & CSD-2 down | CSD-3 handles request; PCF subscriber |
| E00028 | N7 create – all CSDs down | All CSD instances down; returns 504 |
| E00029 | N7 update/delete – CSD-1 down | CSD-2 handles update/delete; PCF subscriber |
| E00030 | N7 update/delete – CSD-1 & CSD-2 down | CSD-3 handles update/delete; PCF subscriber |
| E00031 | N7 update/delete – all CSDs down | All CSD instances down; returns 504 |

Test Case Location: All CSD functional test cases are stored on Windows servers 03, and 04 at the path Documents\Rohit\CSD\SCP_Functional_Tcs.

## 7.2 DRA Functional Test Cases

| Test Case ID | Test Topic | Description |
| --- | --- | --- |
| E00001 | Gx CCR-I Routing | SLF lookup + TAG match routes to k-NPC Mated ME Pair; expects 2001 |
| E00002 | Gx CCR-U Routing | DH AVP match routes to k-NPC Mated ME pair; expects 2001 |
| E00003 | Gx CCR-T Routing | DH AVP match routes to k-NPC Mated ME pair; expects 2001 |
| E00004 | Gx CCR-U/CCR-T (DH Absent) | DH AVP absent; n-DRA performs SLF lookup and routes to DRA; expects 2001 |
| E00005 | Gx CCR-I / One k-NPC down | Selected k-NPC link down; routes to next priority k-NPC; expects 2001 |
| E00006 | Gx CCR-U/CCR-T / One k-NPC down | Unreachable DH k-NPC; routes to next priority k-NPC; expects 2001 |
| E00007 | Gx CCR-I / One DRA down | n-DRA forwards to next available DRA; expects 2001 |
| E00008 | Gx CCR-U/CCR-T / One DRA down | n-DRA forwards to next available DRA; expects 2001 |
| E00009 | Gx CCR-I / One n-DRA down | Clients route to available n-DRA; expects 2001 |
| E00010 | Gx CCR-U/CCR-T / One n-DRA down | Clients route to available n-DRA; expects 2001 |
| E00011 | Gx RAR Routing | DRA forwards RAR to n-DRA; n-DRA routes by DH AVP; expects 2001 |
| E00012 | Gx RAR / DRA down | k-NPC routes to next available DRA; expects 2001 |
| E00013 | Gx RAR / n-DRA down | DRA routes to next available n-DRA; expects 2001 |
| E00014 | Gx RAR / OCSPGW down | n-DRA returns 3002 DIAMETER_UNABLE_TO_DELIVER |
| E00015 | Gx CCR-I / All k-NPCs down | All k-NPCs in Mated Pair down; session fails with 3002 |
| E00016 | Gx CCR-I / All n-DRAs down | All n-DRA instances unreachable; session fails with 3002 |
| E00017 | Gx CCR-U/CCR-T / All k-NPCs down | All k-NPCs in Mated Pair down; session fails with 3002 |
| E00018 | Gx CCR-U/CCR-T / All n-DRAs down | All n-DRA instances unreachable; session fails with 3002 |
| E00019 | Rx-AAR handling | CNDRA receives AAR, routes to PCRF via BSF session; expects 2001 |
| E00020 | Rx-STR handling | CNDRA receives STR, routes by Rx session; session removed post-STR; expects 2001 |
| E00021 | Rx-AAR without BSF/Gx session | No BSF session; routes by RSV; expects 2001 |
| E00022 | Rx-STR without Rx session | No Rx session; routes by RSV; expects 2001 |
| E00023 | Rx session expired | Rx session removed after DRA Destination Mapping Inactivity Duration timer expiry |

Test Case Location: All CSD functional test cases are stored on Windows servers 03, and 04 at the path Documents\Rohit\CSD\DRA_Functional_Tcs.

## 7.3 BSF Functional Test Cases

| Test Case ID | Test Topic | Description |
| --- | --- | --- |
| E00001 | BSF Create message | BSF create flow: NPCv1 -> SCP -> BSF; session stored in db; location header replaced; P1 |
| E00002 | BSF Delete message | BSF delete flow: NPCv1 -> SCP -> BSF; session removed from db; 204 response |
| E00003 | BSF-to-NRF Registration | PUT to /nnrf-nrm/v1/nf-instances; BSF status becomes REGISTERED in Grafana |
| E00004 | BSF-to-NRF De-Registration | DELETE to /nnrf-nrm/v1/nf-instances; BSF status becomes UNREGISTERED in Grafana |
| E00005 | BSF-to-NRF Heartbeat | PATCH /nnrf-nrm/v1/nf-instances; nfStatus remains REGISTERED; 200 OK or 204 |
| E00006 | BSF session expire | BSF session removed from db after IntitalContextExpiry timer expiry |

Test Case Location: All CSD functional test cases are stored on Windows servers 03, and 04 at the path Documents\Rohit\BSF\SCP_Functional_Tcs.

## 7.4 Integration Test Cases

| Test ID | Test Topic | Description |
| --- | --- | --- |
| SCPv Integration_E00001 | K-SCPv to k-NPC N7 Model C call | N7 Model C call successful via SLF/RSV routing |
| SCPv Integration_E00002 | K-SCPv to UDM N10 Model C call | N10 Model C call successful via SLF/RSV routing |
| SCPv Integration_E00003 | K-SCPv to K-UDM N8 Model C call | N8 Model C call successful |
| SCPv Integration_E00004 | K-SCPv to K-AUSF N12 Model C call | N12 Model C call successful |
| SCPv Integration_E00005 | K-SCPv to K-EIR N17 Model C call | N17 Model C call successful |
| BSF Integration_E00006 | BSF to NRF registration/deregistration | BSF registered/deregistered to NRF successfully |
| DRA Integration_E00007 | DRA to NPC – Gx and Rx | NPC peers up; CER/CEA captured successfully |
| DRA Integration_E00008 | DRA to PCSCF – Rx | PCSCF peers up; CER/CEA captured successfully |
| CSD Integration_E00010 | PM data reaching NZE | PM counters on Kibana GUI; file names correct; timestamp in JST |

## 7.5 Overload Handling Test Cases

| Test ID | Test Topic | Expected Result |
| --- | --- | --- |
| Overload_E00001 | Minor alarm verification | Minor alarm raised when pending requests exceeded upper threshold. |
| Overload_E00002 | Major alarm verification | Minor alarm escalated to major after escalation interval. |
| Overload_E00003 | Critical alarm verification | Major alarm escalated to critical after escalation interval. |
| Overload_E00004 | Traffic rejection at critical state | All incoming traffic rejected with HTTP 429 in resource critical state |

Test Case Location: All CSD functional test cases are stored on Windows servers 03 at the path Documents\Rohit\CSD\SCP_Functional_Tcs\tc using CITM IP\load-TPS.

AUSF_UDM Test Case Location: All AUSF_UDM test cases are stored on Windows servers 01 and 02 at the path Desktop\nkrohit.

NPC: Provided comprehensive end-to-end IPSL support to the NPC team for Windows servers 05 and 06, including environment setup, script migration and configuration, IPSL installation, static route configuration, and test case execution guidance for all assigned activities.

Reference Documents: All test case details, execution procedures, and results are documented in the attached Excel file shown below. This spreadsheet contains the complete test case repository for CSD Functional, DRA Functional, BSF Functional, Integration, and Overload Handling test suites.

# Conclusion

This document provides a complete technical handover of the IPSL server setup, script migration, and test case documentation activities for the Nokia KDDI 5GC project. The document covers server assignments, setup procedures, script migration source locations, and the full test case inventory across AUSF_UDM, NPC, CSD Functional, DRA Functional, BSF Functional, Integration, and Overload Handling domains.